IMAGE PROCESSING · FINAL PROJECT Every Coin Counts Counting coins on a textured pad with classical image processing

| Author Yulia Rapana | Course Image Processing summer 2026 | Institution Holon institute of Technology HIT Instructor: Dr. Cornel Lustig |
| --- | --- | --- |

Abstract Counting coins sounds easy until the coins lie on a crumpled white pad, some photos are dark or blurred, coins pile up, and round buttons of the same size are mixed in. A classical image-processing system, with no machine learning, was built to count only the coins. It models the pad and the lighting with a very large median filter, finds round blobs with a multi-scale Laplacian of Gaussian, rejects pad folds with a Hessian shape test and a grayscale opening, separates touching coins with a Hough circle transform, and rejects buttons by their sewing holes (black-hat transform) and by colors no coin has. The coins in 26 phone photographs were counted by hand (328 coins, 12 other round objects), and every circle the program drew was then checked. The program found 294 of the 328 coins (recall 89.6%), and 294 of its 333 detections were real coins (precision 88.3%): 39 false detections and 34 missed coins. On clean photographs it found every coin with a single false detection. 12 photos received the exact count, but the circle-by-circle review showed that 5 of them contained mistakes that cancelled out. False detections came mostly from pad folds in dark photos and from motion blur; missed coins came from dense piles and coins cut by the frame.

Keywords  coin counting · Laplacian of Gaussian · Hessian · Hough circles · morphology · black-hat · precision and recall

1   Introduction

A person looking at a handful of coins on a table can count them at a glance. A computer receives only a grid of pixel values, and many things in a real photograph look round, dark or shiny: folds in the cloth, shadows, the pattern printed on the pad, and in this project buttons that have almost exactly the size of a coin.

The photographs were taken with a phone from above, on a white disposable pad. The pad is the main difficulty: it has a quilted texture and deep fold lines that form a grid across the whole image. A simple threshold sees those folds as objects, and a circle detector finds circles in the texture. On top of that, some photos were taken in a dark room or with a moving hand, so coins become dim or smeared.

The system follows fixed, hand-chosen rules; it does not learn from labelled examples. Every decision was designed to be explainable with techniques from the course: filtering, morphology, derivatives, blob detection and the Hough transform.

Beyond the final number, the aim was to understand what the machine actually detects. For every photo, each circle the program drew was therefore compared with the real scene: which coins it found, which non-coins it mistook for coins, and which coins it missed.

Research question Can a classical image-processing pipeline count the coins in phone photographs taken on a textured, folded pad when the photos may be dark or blurred, coins may touch, overlap or be cut by the frame, and round non-coin objects are mixed in? And which kinds of objects does it wrongly accept or miss?

Clean photographs were expected to be easy and the damaged ones, especially dense piles, to remain the main source of missed coins, since a coin that is mostly hidden gives very little visible evidence.

2   Methods

2.1 The photographs

Twenty-six photographs taken with a phone camera were used (1600 × 900 pixels). All show Israeli coins on the same kind of white pad, photographed from roughly the same height. Before evaluating, the coins and the round non-coin objects in each photograph were counted by hand from the original image (ground_truth.csv). Every photo received an ID (P01–P26) that is used throughout this report. The program is written in Python with OpenCV and NumPy.

| Category | Photos | Coins | What makes it hard |
| --- | --- | --- | --- |
| Clean | 8 | 98 | Sharp, well lit, coins spread over the pad |
| Noisy | 10 | 127 | Dark room and/or motion blur |
| Damaged | 8 | 103 | Dense pile, coins cut by the frame, buttons among coins |
| Total | 26 | 328 | Plus 12 round non-coin objects (buttons) in three photographs |

A synthetic test image with six coloured discs was also generated on a light, noisy background, to check that detection, drawing and counting work before looking at the real photos.

2.2 How success was measured

Comparing only the final number hides a lot: a photo can get the right total while one circle sits on a fold and one coin is missed. So after running the program, every circle it drew was reviewed against the original photo (detection_review.csv) and every object was sorted into one of three groups:

- True positive (TP): a circle on a real coin.

- False positive (FP): a circle on something that is not a coin: a fold, the edge of the pad, a button, or a second circle on the same blurred coin. Each FP was also labelled with its cause.

- False negative (FN): a real coin with no circle.

From these, the report gives precision (TP ÷ all detections: how many circles are right), recall (TP ÷ all real coins: how many coins were found), the mean absolute count error (MAE) and the number of photos with an exact count.

2.3 The system from start to finish

Fig. 1  The complete pipeline. Dark-blue stages prepare the image; green stages find and classify objects. LoG finds blob-shaped candidates, Hough separates touching coins, and the last stage rejects round objects that are not coins.

The key idea is to stop working on raw brightness. The pad is bright and colourless, while every coin, bronze or silver, differs from it either in lightness or in colour. So the program first estimates what the pad would look like without the coins, and then measures how far every pixel is from that estimate. All later stages work on this foreground map, which is why the same settings work for bright and dark photos.

2.4 How each stage works

| 1 Background model Each Lab channel is median-filtered at quarter resolution with a window of 45% of the image width. Coins and even small piles disappear, leaving the pad and the lighting gradient. | 2 Foreground map For each pixel: relative darkness (L_bg − L)/L_bg, half-weighted relative brightness, and colour distance in the a,b plane. The maximum of the three, lightly smoothed, gives a 0–255 score. |
| --- | --- |
| 3 Pad mask Bright, colourless background region connected to the image centre. Knees, floor or bed around the pad are ignored. | 4 Multi-scale LoG −σ²∇²G on the foreground map for 12 radii (2.5–5.5% of width), σ = r/√2, max over scales. Local maxima are coin candidates with their radius. |
| 5 Blob verification Hessian eigenvalue ratio ≥ 0.25 (round, not a ridge); peak ≥ 20% of a typical coin; radius ≥ 70% of the typical coin; ≥ 40% of the response survives a grayscale opening; no stronger coin within 0.95 r. | 6 Hough circles HOUGH_GRADIENT_ALT on the median-blurred gray image with radius 0.6–1.3 × coin radius. Circles must lie on the pad and differ from the background. LoG blobs not explained by any circle are added. |
| 7 Non-coin test Black-hat (closing − image) highlights small dark spots: 2–4 similar round spots around the centre are sewing holes. Strongly blue/purple or pink/red objects are also rejected. | 8 Final count Green circles are counted as coins; red circles with an X are round non-coin objects and are not counted. Annotated images and results.csv are saved. |

Fig. 2  The main stages for one clean photograph (P05). The foreground map (2) already separates coins from the pad, but the fold lines are still visible. They produce many LoG peaks (5); the verification stage rejects all of them and exactly 14 coins remain (6).

| Self-calibrating setup No fixed pixel sizes or gray levels: radii scale with the image width and every threshold is relative to the coins measured in the same photo. | Input requirements Top view, a bright colourless pad, coins 5–11% of the image width, and some part of every coin visible. |
| --- | --- |

3   Results

3.1 The synthetic check

| Synthetic test 6 discs expected, 6 detected. The synthetic image is processed by exactly the same code as the photographs. It confirms that background modelling, blob detection, drawing and counting work end to end. | Why a synthetic image In a synthetic image the right answer is known with certainty. Any error there would be a bug, not a hard photo. |
| --- | --- |

Fig. 3  The synthetic input and its result.

3.2 The overall result

| 89.6% Recall 294 of 328 real coins found | 88.3% Precision 294 of 333 detections were coins | 39 / 34 FP / FN False detections / missed coins | 12/26 Exact photos only 7 with no mistake at all |
| --- | --- | --- | --- |

Results by category

| Category | Coins | Counted | TP | FP | FN | Precision | Recall | Exact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clean | 98 | 99 | 98 | 1 | 0 | 99.0% | 100.0% | 7/8 |
| Noisy | 127 | 142 | 116 | 26 | 11 | 81.7% | 91.3% | 3/10 |
| Damaged | 103 | 92 | 80 | 12 | 23 | 87.0% | 77.7% | 2/8 |
| All photos | 328 | 333 | 294 | 39 | 34 | 88.3% | 89.6% | 12/26 |

The three categories fail in different ways. On clean photographs the program found all 98 coins with a single false detection. In the noisy set the main problem is false detections (26 FP against 11 FN): in dark and blurred photos the program "sees" coins on folds and counts smeared coins twice. In the damaged set the problem is reversed (12 FP against 23 FN): coins hidden in piles or cut by the frame are missed.

3.3 Results by photograph

For every photo: the true number of coins, the number the program counted, and the circle-by-circle review. Counted = TP + FP and True = TP + FN.

| Photo | Category | True | Counted | TP | FP | FN | Count error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P01 | clean | 14 | 14 | 14 | 0 | 0 | 0 |
| P02 | clean | 9 | 10 | 9 | 1 | 0 | +1 |
| P03 | clean | 7 | 7 | 7 | 0 | 0 | 0 |
| P04 | clean | 14 | 14 | 14 | 0 | 0 | 0 |
| P05 | clean | 14 | 14 | 14 | 0 | 0 | 0 |
| P06 | clean | 14 | 14 | 14 | 0 | 0 | 0 |
| P07 | clean | 14 | 14 | 14 | 0 | 0 | 0 |
| P08 | clean | 12 | 12 | 12 | 0 | 0 | 0 |
| P09 | noisy | 13 | 12 | 12 | 0 | 1 | -1 |
| P10 | noisy | 13 | 13 | 11 | 2 | 2 | 0 |
| P11 | noisy | 13 | 14 | 13 | 1 | 0 | +1 |
| P12 | noisy | 12 | 13 | 12 | 1 | 0 | +1 |
| P13 | noisy | 12 | 13 | 10 | 3 | 2 | +1 |
| P14 | noisy | 12 | 22 | 10 | 12 | 2 | +10 |
| P15 | noisy | 12 | 12 | 11 | 1 | 1 | 0 |
| P16 | noisy | 12 | 14 | 11 | 3 | 1 | +2 |
| P17 | noisy | 14 | 15 | 14 | 1 | 0 | +1 |
| P18 | noisy | 14 | 14 | 12 | 2 | 2 | 0 |
| P19 | damaged | 14 | 10 | 10 | 0 | 4 | -4 |
| P20 | damaged | 14 | 17 | 14 | 3 | 0 | +3 |
| P21 | damaged | 9 | 9 | 7 | 2 | 2 | 0 |
| P22 | damaged | 13 | 9 | 8 | 1 | 5 | -4 |
| P23 | damaged | 14 | 12 | 11 | 1 | 3 | -2 |
| P24 | damaged | 14 | 14 | 11 | 3 | 3 | 0 |
| P25 | damaged | 14 | 13 | 11 | 2 | 3 | -1 |
| P26 | damaged | 11 | 8 | 8 | 0 | 3 | -3 |
| Total |  | 328 | 333 | 294 | 39 | 34 | 35 abs. |

Fig. 4  False detections (up) and missed coins (down) for every photo. Clean photos are almost free of mistakes; dark photos mostly add false detections (the motion-blurred P14 alone has 12); damaged photos mostly lose coins.

3.4 An exact count is not always a correct count

12 photos received the exact number of coins, but only 7 photos had no mistake at all. In 5 "exact" photos (P10, P15, P18, P21, P24) a false detection and a missed coin cancelled each other out. All 7 exact clean photos were truly perfect; every exact photo in the noisy and damaged sets hid mistakes. This is the main reason every circle was reviewed instead of comparing totals only.

Fig. 12  P10: the program counted 13 coins and there are 13, yet two circles lie on empty pad (FP, red) and two faint coins have no circle (FN, blue).

3.5 What the machine mistook for a coin

Every false detection was labelled with its cause. The largest group, 22 of 39, are pad folds and dark patches of background, almost all in the dark photos where the contrast of a real coin is close to that of a fold. 8 are second circles on a coin smeared by motion blur, 7 lie on the white hem at the top or bottom edge of the pad, and 2 are grey buttons that passed the non-coin test.

Fig. 11  Causes of the 39 false positives.

3.6 Examples with no mistakes

Fig. 5  Three clean photos with every coin found and no false detection. Coins that touch each other are counted separately, and the fold grid of the pad produces no circles.

3.7 How each stage reduced the error

The system was built step by step and measured on all 26 photographs after every change, against a preliminary count of the coins. This shows which idea solved which problem.

Fig. 6  Mean absolute count error of each development version on the same 26 photos (preliminary count).

- Threshold + watershed (MAE 7.38). The first version thresholded the foreground map and split touching blobs with a watershed. The pad folds passed the threshold, and in dark photos the threshold missed coins completely.

- LoG blobs (5.81). Looking for blob-shaped peaks instead of thresholded regions helped in dark photos, but fold intersections also look like blobs.

- + Hessian shape test (2.54). The biggest single improvement. A fold is a ridge: one eigenvalue of the Hessian is large and the other near zero. A coin is round, so both are similar. Rejecting ridge-like peaks removed most false detections.

- + Hough circles (1.19). LoG merges coins that touch into one blob. The circle Hough transform uses the visible arc of each coin, so it separates them.

- + Non-coin classifier (1.27). The final step targets buttons rather than the coin count; it made the hole test more sensitive and changed the coin MAE only slightly.

3.8 The pad folds

Figure 7 shows why the Hessian test matters (P23): LoG finds dozens of peaks along the fold lines, and the Hessian test rejected 63 of them in this photo alone. The folds that still get through (Section 3.5) are the ones in dark photos, where a fold and a coin look almost the same.

Fig. 7  Red: LoG peaks rejected as ridge-like (fold lines). Green: the final detections. Almost all rejected peaks lie exactly on the folds.

3.9 Round objects that are not coins

Three photographs contain 12 buttons among the coins. 9 of them were rejected (red circle with an X) and not counted, and no coin was rejected by mistake. Two grey buttons were counted as coins (P20 and P22): they have no telling colour and their holes were not found. One dark-blue button in P22 lies under other coins and was not detected at all, so it did not affect the count.

Fig. 8  The three photographs with buttons (P20, P21, P22). Blue, dark-navy and pink buttons are recognised by colour, grey ones only by their holes.

Fig. 9  The black-hat transform keeps only small dark details. On a button the sewing holes light up as a symmetric group near the centre; on a coin the embossing gives no such group.

3.10 Where the largest errors came from

Fig. 10  P14: strong motion blur smears each coin into several blobs (22 counted, 12 real). P19: a dense pile where several coins are almost completely hidden (10 of 14 found). P26: coins cut by the image frame are only half-circles (8 of 11).

4   Discussion and conclusions

What worked

On clean photographs the program found all 98 coins with a single false detection, including coins that touch each other on a pad covered with folds. Every decision is made on the foreground map, and every threshold is relative to the coins in the same photo, so the same settings also found 91.3% of the coins in the dark and blurred set. Two detectors were needed: LoG is robust to blur but merges touching coins, while Hough separates touching coins but needs sharp edges.

Two different kinds of failure

The review showed that the program fails in two opposite ways, depending on the photo. In dark and blurred photos it accepts too much: folds and smeared patches pass as coins (precision 81.7%). In piles and at the image frame it accepts too little: a coin that shows only a crescent or half a disk gives neither a LoG blob nor enough arc for Hough (recall 77.7%). A single threshold cannot fix both: loosening it would find more hidden coins but add more folds in dark photos.

What the folds showed

Brightness alone is not enough to decide what is an object: the folds of the pad are as different from the pad as a coin is. What separates them is shape, measured locally by the Hessian: a fold is long and thin, a coin is round. This shape test halved the error during development, and the folds that still pass are the ones in photos too dark to show their shape.

What the non-coin objects showed

Size and roundness cannot separate a button from a coin. Colour (no coin is blue or pink) and sewing holes, isolated by the black-hat transform, can. A grey button is the hard case: only its small holes reveal it, and they may not survive blur or low resolution.

Reading the measures

Comparing totals alone would have suggested that 12 of 26 photos were correct. Checking every circle showed that only 7 were free of mistakes, and that 39 false detections and 34 missed coins partly cancel in the total (333 counted against 328 real). Precision and recall describe the detector; the count error describes only the final number.

What could be improved

- Estimate the direction of motion blur (e.g. from the elongation of the strongest blobs) and merge detections lying along it.

- Exclude the white hem of the pad from the pad mask, which would remove the edge false positives.

- Detect coins cut by the frame with partial-circle fitting near the image border.

- Use the colour of the rim in addition to holes to catch grey buttons.

- A fully hidden coin cannot be counted from a single photo; a second view would be needed.

Final conclusion

The project answered its research question for clean and moderately difficult photos. With classical tools only (a median background model, a Lab foreground map, multi-scale LoG, a Hessian shape test, grayscale morphology, a Hough transform and a black-hat hole detector), the program found 89.6% of the coins with 88.3% precision and rejected 9 of 12 buttons, with no training data and about 1.2 seconds per photo. The circle-by-circle review was as important as the algorithm itself: it revealed that dark photos produce false coins while piles hide real ones, a difference that the total count could not show.

A   Full code

The project has two Python files. coin_counter.py holds the whole pipeline. run.py runs it on every image in the data folder, saves the annotated images and results.csv, and prints the evaluation. All figures in this report are produced by report/make_figures.py.

Minimal working example python run.py # all photos in data/ python run.py "data/photo.jpeg" # a single photo

Listing 1: coin_counter.py

"""

coin_counter.py — classical (non-deep-learning) coin counter.

Pipeline

--------

1. Background model : the pad and the lighting are estimated with a very large

median filter (objects much smaller than the window vanish).

2. Foreground map   : per-pixel difference from that background in Lab space —

lightness difference (both darker bronze and brighter

silver coins) and colour difference.

3. Pad mask         : detections are accepted only on the pad (bright, colourless

region) — ignores knees, floor, bed etc. around it.

4. Blob detection   : multi-scale, scale-normalised Laplacian of Gaussian (LoG).

A disk of radius r gives a strong peak at scale σ = r/√2;

thin pad folds give weak responses. Touching coins still

give separate peaks.

5. Verification     : each LoG candidate must be (a) round, not ridge-like

(Hessian eigenvalue ratio), (b) a strong enough peak,

(c) close to the typical coin size of the image, (d)

"survive" a grayscale opening (thin structures don't),

(e) not too close to an already accepted, stronger one.

6. Hough circles    : circle Hough transform with the radius range fixed around

the coin size found in step 5. Separates coins in piles

and overlapping coins, which LoG merges. LoG blobs not

explained by any circle are added (blurred / dark photos

have no sharp edges for Hough).

7. Classification   : coin vs. button. Buttons are recognised by their sewing

holes (2-4 small dark round spots near the centre) and by

colours coins never have (blue, pink/red).

"""

from dataclasses import dataclass, field

import cv2

import numpy as np

# ── Parameters ─────────────────────────────────────────────────────

# Coin radius range as a fraction of the image width (photos are taken from

# roughly the same height, so coins are 5-11 % of the width across).

R_MIN_FRAC = 0.025

R_MAX_FRAC = 0.055

N_SCALES = 12

BG_WINDOW_FRAC = 0.45       # median-filter window for the background, × image width

PEAK_REL = 0.20             # min LoG response, × median response of the strong peaks

SIZE_REL = 0.70             # min radius, × typical coin radius

MIN_DIST_REL = 0.95         # min distance between two coins, × typical coin radius

SURVIVAL_MIN = 0.40         # min fraction of the response left after opening

HOUGH_PERFECTNESS = 0.75    # HOUGH_GRADIENT_ALT param2: how perfect a circle must be

HOUGH_MIN_DIST = 0.6        # min distance between Hough centres, × coin radius

HOUGH_FG_REL = 0.45         # Hough circle inside must reach this × typical coin contrast

MERGE_DIST = 0.9            # LoG blob closer than this to a Hough circle = same coin

ISOTROPY_MIN = 0.25         # min Hessian eigenvalue ratio (1 = round, 0 = line)

# ── Data structures ────────────────────────────────────────────────

@dataclass

class DetectedObject:

label: str                 # "coin" | "button"

center: tuple              # (x, y)

radius: float

response: float            # detection strength (LoG peak / mean foreground for Hough)

holes: int

hue: float

saturation: float

reason: str = ""

@dataclass

class CountResult:

coins: int

buttons: int

objects: list = field(default_factory=list)

coin_radius: float = 0.0

# ── Image I/O ──────────────────────────────────────────────────────

# cv2.imread / cv2.imwrite fail on Windows paths with non-ASCII characters

# (e.g. a Hebrew user name), so read/write through NumPy instead.

def load_image(path) -> np.ndarray:

img = cv2.imdecode(np.fromfile(str(path), dtype=np.uint8), cv2.IMREAD_COLOR)

if img is None:

raise ValueError(f"Cannot read image: {path}")

return img

def save_image(path, img: np.ndarray) -> None:

ext = str(path).rsplit(".", 1)[-1]

ok, buf = cv2.imencode("." + ext, img)

if not ok:

raise ValueError(f"Cannot encode image: {path}")

buf.tofile(str(path))

# ── Helpers ────────────────────────────────────────────────────────

def _disk(r: float) -> np.ndarray:

r = max(1, int(round(r)))

return cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (2 * r + 1, 2 * r + 1))

def _disk_mask(shape, cx, cy, r) -> np.ndarray:

m = np.zeros(shape[:2], np.uint8)

cv2.circle(m, (int(round(cx)), int(round(cy))), max(1, int(round(r))), 1, -1)

return m.astype(bool)

# ── 1. Background model ────────────────────────────────────────────

def background(lab: np.ndarray) -> np.ndarray:

"""Per-channel median filter at 1/4 resolution with a window much bigger than a coin pile."""

h, w = lab.shape[:2]

small = cv2.resize(lab, (w // 4, h // 4), interpolation=cv2.INTER_AREA)

k = int(BG_WINDOW_FRAC * w / 4) | 1           # odd kernel size

bg = np.dstack([cv2.medianBlur(small[..., c], k) for c in range(3)])

return cv2.resize(bg, (w, h), interpolation=cv2.INTER_LINEAR).astype(np.float32)

def _center_level(x: np.ndarray) -> float:

h, w = x.shape[:2]

return float(np.median(x[h // 4: 3 * h // 4, w // 4: 3 * w // 4]))

# ── 2. Foreground map ──────────────────────────────────────────────

def foreground_map(lab: np.ndarray, bg: np.ndarray) -> np.ndarray:

"""0-255 score: how much each pixel differs from the local background."""

L, a, b = cv2.split(lab.astype(np.float32))

bgL, bga, bgb = cv2.split(bg)

# Relative lightness difference in both directions (bronze coins are darker

# than the pad, polished silver ones brighter). Normalising by the

# background level handles dark photos; the floor stops near-black image

# borders from amplifying sensor noise.

denom = np.maximum(bgL, 0.5 * _center_level(bgL))

darker = np.clip(bgL - L, 0, None) / denom * 510.0

# Brighter-than-pad counts half: white pad folds are brighter too

brighter = np.clip(L - bgL, 0, None) / denom * 255.0

chroma = np.sqrt((a - bga) ** 2 + (b - bgb) ** 2) * 4.0

fmap = cv2.GaussianBlur(np.maximum(np.maximum(darker, brighter), chroma), (0, 0), 2)

return np.clip(fmap, 0, 255).astype(np.uint8)

# ── 3. Pad mask ────────────────────────────────────────────────────

def pad_mask(bg: np.ndarray) -> np.ndarray:

"""The pad = bright, colourless background region connected to the image centre."""

bgL = bg[..., 0]

bgC = np.hypot(bg[..., 1] - 128, bg[..., 2] - 128)

pad = (bgL > 0.8 * _center_level(bgL)) & (bgC < _center_level(bgC) + 5)

pad = pad.astype(np.uint8)

n, labels, stats, _ = cv2.connectedComponentsWithStats(pad)

if n <= 1:

return np.ones_like(pad, bool)

h, w = pad.shape

lab_c = labels[h // 2, w // 2]

keep = lab_c if lab_c != 0 else 1 + int(np.argmax(stats[1:, cv2.CC_STAT_AREA]))

return labels == keep

# ── 4. Multi-scale LoG blob detection ──────────────────────────────

def log_blobs(fmap: np.ndarray, radii: np.ndarray):

"""

Scale-normalised LoG, maximised over scales.

Returns (response, best_radius, smoothed-per-scale). −σ²·∇²G is positive at

the centre of bright blobs; σ = r/√2 matches a disk of radius r.

"""

f = fmap.astype(np.float32)

best = np.full(f.shape, -np.inf, np.float32)

best_r = np.zeros(f.shape, np.float32)

smoothed = {}

for r in radii:

s = r / np.sqrt(2)

g = cv2.GaussianBlur(f, (0, 0), s)

smoothed[round(float(r), 1)] = g

resp = -(s * s) * cv2.Laplacian(g, cv2.CV_32F, ksize=3)

better = resp > best

best[better] = resp[better]

best_r[better] = r

return best, best_r, smoothed

def blob_isotropy(g: np.ndarray, cx: float, cy: float, r: float) -> float:

"""

Ratio of the Hessian eigenvalues (small/large) at the blob centre.

≈1 for round blobs, ≈0 for ridges such as pad folds (same idea as SIFT's

edge-response elimination).

"""

d = max(1, int(round(r / 2)))

x, y = int(round(cx)), int(round(cy))

h, w = g.shape

if not (d <= x < w - d and d <= y < h - d):

return 0.0

c = g[y, x]

dxx = g[y, x + d] + g[y, x - d] - 2 * c

dyy = g[y + d, x] + g[y - d, x] - 2 * c

dxy = (g[y + d, x + d] - g[y + d, x - d] - g[y - d, x + d] + g[y - d, x - d]) / 4

tr, det = dxx + dyy, dxx * dyy - dxy * dxy

disc = np.sqrt(max(tr * tr / 4 - det, 0))

l1, l2 = tr / 2 + disc, tr / 2 - disc          # both negative for a bright blob

big, small = max(abs(l1), abs(l2)), min(abs(l1), abs(l2))

if big == 0 or l1 > 0:                         # not a bright blob

return 0.0

return float(small / big)

def find_candidates(resp, best_r, pad, r_min):

"""Local maxima of the LoG response inside the pad, strongest first."""

peaks = (resp == cv2.dilate(resp, _disk(0.5 * r_min))) & (resp > 10) & pad

ys, xs = np.nonzero(peaks)

order = np.argsort(-resp[ys, xs])

return [(float(xs[i]), float(ys[i]), float(resp[ys[i], xs[i]]), float(best_r[ys[i], xs[i]]))

for i in order]

def typical_radius(cands) -> tuple:

"""Coin radius and reference strength from the strongest peaks (coins dominate them)."""

top = [c for c in cands if c[2] >= 0.5 * cands[0][2]][:15]

return float(np.median([c[3] for c in top])), float(np.median([c[2] for c in top]))

# ── 5. Verification ────────────────────────────────────────────────

def survival_maps(fmap: np.ndarray, coin_r: float):

"""

closed : grayscale closing fills the inside of ring-like silver coins.

opened : grayscale opening with a half-coin disk removes everything

thinner than that (folds, seams); coins survive almost unchanged.

"""

closed = cv2.morphologyEx(fmap, cv2.MORPH_CLOSE, _disk(0.45 * coin_r))

opened = cv2.morphologyEx(closed, cv2.MORPH_OPEN, _disk(0.5 * coin_r))

return closed.astype(np.float32), opened.astype(np.float32)

def survival(closed, opened, cx, cy, r) -> float:

m = _disk_mask(closed.shape, cx, cy, 0.5 * r)

return float(opened[m].mean() / max(closed[m].mean(), 1.0))

# ── 5b. Hough circles (separates touching / overlapping coins) ─────

def hough_circles(gray: np.ndarray, coin_r: float, pad: np.ndarray,

fmap: np.ndarray, min_fmap: float) -> list:

"""

Circle Hough transform with the radius range fixed around the coin size.

Every visible arc of a coin votes for its centre, so coins in a pile or

partly covered by another coin are still found. Needs sharp edges — in

dark / blurred photos it simply finds nothing and LoG does the work.

A circle is kept only if it lies on the pad and its inside differs from

the background (rejects circles "seen" in the pad texture).

"""

g = cv2.medianBlur(gray, 5)

found = cv2.HoughCircles(

g, cv2.HOUGH_GRADIENT_ALT, dp=1.5, minDist=HOUGH_MIN_DIST * coin_r,

param1=150, param2=HOUGH_PERFECTNESS,

minRadius=int(0.6 * coin_r), maxRadius=int(1.3 * coin_r),

)

if found is None:

return []

circles = []

for cx, cy, r in found[0]:

x, y = int(cx), int(cy)

if not (0 <= x < pad.shape[1] and 0 <= y < pad.shape[0]) or not pad[y, x]:

continue

m = _disk_mask(fmap.shape, cx, cy, 0.7 * r)

if fmap[m].mean() < min_fmap:

continue

# HOUGH_GRADIENT_ALT can return (nearly) concentric circles for one coin

if any(np.hypot(cx - c[0], cy - c[1]) < 0.3 * coin_r for c in circles):

continue

circles.append((float(cx), float(cy), float(fmap[m].mean()), float(r)))

return circles

def merge_detections(hough: list, log: list, coin_r: float) -> list:

"""

Hough circles are precise, so they come first; a LoG blob is added only if

no Hough circle already explains it (blurred / dark coins without edges).

"""

merged = list(hough)

for c in log:

if all(np.hypot(c[0] - m[0], c[1] - m[1]) > MERGE_DIST * coin_r for m in merged):

merged.append(c)

return merged

# ── 6. Button detection ────────────────────────────────────────────

def count_holes(gray: np.ndarray, cx: float, cy: float, r: float) -> int:

"""

Sewing holes: 2-4 small, dark, round spots of similar size, grouped

symmetrically around the centre. Black-hat (closing − image) highlights

dark details smaller than the structuring element.

"""

pad = int(1.2 * r)

x0, y0 = max(0, int(cx) - pad), max(0, int(cy) - pad)

x1, y1 = min(gray.shape[1], int(cx) + pad), min(gray.shape[0], int(cy) + pad)

patch = gray[y0:y1, x0:x1]

lcx, lcy = cx - x0, cy - y0

inner = _disk_mask(patch.shape, lcx, lcy, 0.5 * r)

blackhat = cv2.morphologyEx(patch, cv2.MORPH_BLACKHAT, _disk(max(3, 0.25 * r)))

vals = blackhat[inner]

if vals.size == 0:

return 0

# Relative to the darkest detail: holes are the strongest dark spots on a button

thr = max(25, 0.45 * float(vals.max()))

spots = ((blackhat >= thr) & inner).astype(np.uint8)

contours, _ = cv2.findContours(spots, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_NONE)

min_a, max_a = 0.004 * np.pi * r * r, 0.05 * np.pi * r * r

holes = []

for c in contours:

a = cv2.contourArea(c)

per = cv2.arcLength(c, True)

if not (min_a <= a <= max_a) or per == 0:

continue

if 4 * np.pi * a / per ** 2 < 0.5:

continue

m = cv2.moments(c)

holes.append((m["m10"] / m["m00"], m["m01"] / m["m00"], a))

if not 2 <= len(holes) <= 4:

return 0

areas = np.array([h[2] for h in holes])

if areas.max() > 3 * areas.min():

return 0

hx = np.mean([h[0] for h in holes])

hy = np.mean([h[1] for h in holes])

if np.hypot(hx - lcx, hy - lcy) > 0.25 * r:

return 0

return len(holes)

def object_colour(hsv: np.ndarray, cx, cy, r) -> tuple:

"""Circular-mean hue and median saturation of the object's core."""

m = _disk_mask(hsv.shape, cx, cy, 0.6 * r)

h, s = hsv[..., 0][m], hsv[..., 1][m]

ang = h.astype(np.float32) * np.pi / 90.0          # OpenCV hue is 0..180

hue = (np.degrees(np.arctan2(np.sin(ang).mean(), np.cos(ang).mean())) / 2.0) % 180

return float(hue), float(np.median(s))

def classify(hue: float, sat: float, holes: int) -> tuple:

"""

Coins are gold (yellow-orange) or silver (grey). Anything else is a button.

"""

if sat > 60 and 95 <= hue <= 165:

return "button", "blue/purple colour"

if sat > 70 and (hue >= 165 or hue <= 6):

return "button", "pink/red colour"

if holes >= 2:

return "button", f"{holes} holes"

return "coin", ""

# ── Full pipeline ──────────────────────────────────────────────────

def count_coins(img: np.ndarray) -> CountResult:

h, w = img.shape[:2]

lab = cv2.cvtColor(img, cv2.COLOR_BGR2LAB)

bg = background(lab)

fmap = foreground_map(lab, bg)

pad = pad_mask(bg)

radii = np.linspace(R_MIN_FRAC * w, R_MAX_FRAC * w, N_SCALES)

resp, best_r, smoothed = log_blobs(fmap, radii)

cands = find_candidates(resp, best_r, pad, radii[0])

# Drop ridge-like peaks (pad folds) before estimating the coin size

cands = [c for c in cands if blob_isotropy(smoothed[round(c[3], 1)], c[0], c[1], c[3]) >= ISOTROPY_MIN]

if not cands:

return CountResult(coins=0, buttons=0)

coin_r, ref = typical_radius(cands)

closed, opened = survival_maps(fmap, coin_r)

accepted = []

for cx, cy, val, r in cands:

if val < PEAK_REL * ref or r < SIZE_REL * coin_r:

continue

if any(np.hypot(cx - ax, cy - ay) < MIN_DIST_REL * coin_r for ax, ay, _, _ in accepted):

continue

if survival(closed, opened, cx, cy, coin_r) < SURVIVAL_MIN:

continue

accepted.append((cx, cy, val, r))

hsv = cv2.cvtColor(img, cv2.COLOR_BGR2HSV)

gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

# Foreground level of the accepted blobs → minimum for a Hough circle

levels = [fmap[_disk_mask(fmap.shape, cx, cy, 0.7 * coin_r)].mean() for cx, cy, _, _ in accepted]

min_fmap = HOUGH_FG_REL * float(np.median(levels)) if levels else 30.0

circles = hough_circles(gray, coin_r, pad, fmap, min_fmap)

detections = merge_detections(circles, accepted, coin_r)

result = CountResult(coins=0, buttons=0, coin_radius=coin_r)

for cx, cy, val, r in detections:

hue, sat = object_colour(hsv, cx, cy, r)

holes = count_holes(gray, cx, cy, r)

label, reason = classify(hue, sat, holes)

result.objects.append(DetectedObject(label, (cx, cy), r, val, holes, hue, sat, reason))

result.coins = sum(o.label == "coin" for o in result.objects)

result.buttons = sum(o.label == "button" for o in result.objects)

return result

def draw_result(img: np.ndarray, result: CountResult) -> np.ndarray:

"""Annotated copy: green circle = coin, red circle + X = button."""

out = img.copy()

for o in result.objects:

x, y, r = int(o.center[0]), int(o.center[1]), int(o.radius)

if o.label == "coin":

cv2.circle(out, (x, y), r, (0, 200, 0), 3)

else:

cv2.circle(out, (x, y), r, (0, 0, 255), 3)

d = int(r * 0.6)

cv2.line(out, (x - d, y - d), (x + d, y + d), (0, 0, 255), 3)

cv2.line(out, (x - d, y + d), (x + d, y - d), (0, 0, 255), 3)

text = f"Coins: {result.coins}   Buttons: {result.buttons}"

cv2.rectangle(out, (0, 0), (out.shape[1], 60), (0, 0, 0), -1)

cv2.putText(out, text, (15, 42), cv2.FONT_HERSHEY_SIMPLEX, 1.2, (255, 255, 255), 2)

return out

Listing 2: run.py

"""

run.py — count coins in every image of the data folder and evaluate.

Usage:

python run.py                      # all images under data/

python run.py path/to/image.jpg    # a single image

python run.py --data data --gt ground_truth.csv --review detection_review.csv --out results

Outputs (in --out):

<image name>.jpg   annotated image (green = counted coin, red X = round object rejected as not a coin)

results.csv        machine count vs. true count per image (+ TP / FP / FN from the manual review)

The manual review (detection_review.csv) was made by checking every circle the program drew:

TP = circle on a real coin, FP = circle on something that is not a coin, FN = real coin without a circle.

It describes this version of coin_counter.py; if the code changes and the machine count of a photo no

longer matches the reviewed count, that photo's TP/FP/FN are left empty.

"""

import argparse

import csv

import sys

from pathlib import Path

from coin_counter import count_coins, draw_result, load_image, save_image

IMAGE_EXTS = {".jpg", ".jpeg", ".png", ".bmp"}

def read_csv(path: Path, key: str) -> dict:

"""key column -> row dict. Skips Excel's 'sep=,' hint line if present."""

if not path.is_file():

return {}

with open(path, encoding="utf-8-sig") as f:

lines = [l for l in f if not l.startswith("sep=")]

return {row[key]: row for row in csv.DictReader(lines)}

def to_int(value):

try:

return int(value)

except (TypeError, ValueError):

return None

def fmt(v, spec=""):

return "" if v is None else format(v, spec)

def main() -> None:

# Windows console defaults to a legacy code page; make printing safe

sys.stdout.reconfigure(encoding="utf-8", errors="replace")

ap = argparse.ArgumentParser(description="Classical coin counter")

ap.add_argument("images", nargs="*", help="image files (default: everything under --data)")

ap.add_argument("--data", default="data", help="folder with images (searched recursively)")

ap.add_argument("--gt", default="ground_truth.csv", help="ground-truth CSV (true coin counts)")

ap.add_argument("--review", default="detection_review.csv", help="manual circle-by-circle review")

ap.add_argument("--out", default="results", help="output folder")

args = ap.parse_args()

if args.images:

paths = [Path(p) for p in args.images]

else:

paths = sorted(p for p in Path(args.data).rglob("*") if p.suffix.lower() in IMAGE_EXTS)

if not paths:

sys.exit(f"No images found in {args.data}")

gt = read_csv(Path(args.gt), "filename")

review = read_csv(Path(args.review), "filename")

out_dir = Path(args.out)

out_dir.mkdir(exist_ok=True)

rows = []

print(f"{'image':<48} {'category':<9} {'coins':>5} {'true':>5} {'error':>6} {'TP':>4} {'FP':>4} {'FN':>4}")

for path in paths:

result = count_coins(load_image(path))

save_image(out_dir / (path.stem + ".jpg"), draw_result(load_image(path), result))

g = gt.get(path.name, {})

category = g.get("category") or (path.parent.name if path.parent.name != Path(args.data).name else "")

true = to_int(g.get("true_count"))

err = None if true is None else result.coins - true

# Review applies only if the machine still reports the reviewed count

rv = review.get(path.name, {})

tp = fp = fn = None

if to_int(rv.get("machine_coins")) == result.coins:

tp, fp, fn = to_int(rv.get("tp")), to_int(rv.get("fp")), to_int(rv.get("fn"))

rows.append(dict(filename=path.name, category=category, machine_coins=result.coins,

true_count=fmt(true), error=fmt(err), tp=fmt(tp), fp=fmt(fp), fn=fmt(fn)))

print(f"{path.name:<48} {category:<9} {result.coins:>5} {fmt(true):>5} {fmt(err, '+d'):>6} "

f"{fmt(tp):>4} {fmt(fp):>4} {fmt(fn):>4}")

with open(out_dir / "results.csv", "w", newline="", encoding="utf-8-sig") as f:

w = csv.DictWriter(f, fieldnames=list(rows[0].keys()))

w.writeheader()

w.writerows(rows)

# ── Evaluation (only images that have a true count) ─────────────

scored = [r for r in rows if r["error"] != ""]

if scored:

print("\nEvaluation")

groups = {"all": scored}

for r in scored:

groups.setdefault(r["category"] or "-", []).append(r)

for name, grp in groups.items():

errs = [int(r["error"]) for r in grp]

line = (f"  {name:<9} images={len(grp):>2}  MAE={sum(map(abs, errs)) / len(grp):.2f}  "

f"exact={sum(e == 0 for e in errs)}/{len(grp)}")

reviewed = [r for r in grp if r["tp"] != ""]

if len(reviewed) == len(grp):

tp, fp, fn = (sum(int(r[k]) for r in grp) for k in ("tp", "fp", "fn"))

line += (f"  TP={tp} FP={fp} FN={fn}  "

f"precision={tp / (tp + fp):.1%}  recall={tp / (tp + fn):.1%}")

print(line)

print(f"\nAnnotated images and results.csv saved to: {out_dir.resolve()}")

if __name__ == "__main__":

main()

B   Original test photographs

All 26 photographs with their ID, category and the number of coins counted by hand.